Preliminaries to using Fourier transform.  Understanding definitions of stresses

The equation that the stresses satisfy if we define a stress function is the biharmonic equation 4

Stress tensor has two unit vectors: one for direction of force component/one perpendicular to surface

If we speak of a traction vector on a surface, ty, take dot product of stress tensor with normal to that surface.

On a surface there are 3 stresses.  For problems in plane stress, where no variations with z, yz =0.  To find a particular stress, e.g., yz , take the dot product of the stress tensor with the unit vectors ey and ez

Einsteinian notation says that if two elements have the same subscript, it represents a summation