G-CODES FOR ANILAM MILLING  MACHINE

G00	RAPID MOTION

G01	FEED MOTION

G70	INCHES MOVEMENT

G71	METRIC MOVEMENT

G90	ABSOLUTE MOVEMENT

G91	INCREMENT MOVEMENT

G02	ARC CW X(DEST) Y (DEST)   I(CENTER X)  J (CENTER Y) *when swing is over 90-Deg

G02	ARC CW X(DEST) Y(DEST) R(RADIUS) *when swing is less than 90-Deg

G03	ARC CCW X(DEST) Y(DEST) I(CENTER X)  J(CENTER Y) *when swing is over 90-Deg

G03	ARC CCW X-(DEST) Y-(DEST) R-(RADIUS) *when swing is Less than 90-Deg

G40	CANCEL TOOL COMPENSATION

G41	CUTTER “LEFT” TOOL COMPENSATION

G42	CUTTER “RIGHT” TOOL COMPENSATION

G4	Tn	DWELL TIME IN SECONDS

S	SPINDLE SPEED (EX: S1500 = 1500 RPM)

M8	(FLOOD COOLANT ON)

M9	(FLOOD COOLANT OFF)

(ADDITIONAL CODES)

T 0		CANCELs ALL PREVIOUS TOOL OFFSETS AND DIA. COMPENSATION

T		TOOL NUMBER DEFINED IN TOOL PAGE (Dia. And Length offsets) (ex: T5)

F		FEED IN INCHES PER MINUTE (ipm) (ex: F10.)

On		SUBROUTINE Label ( Ex: O5) note: letter O not zero

M99		END OF SUB ROUTINE

M98 Pn	CALL SUB ROUTINE BY NUMBER (Ex: M98 P5)

M2		END OF MAIN PROGRAM

TLO  (Tool Length Offset) : The distance from the tool tip to the “Part Z-zero” (Distance taken when Tool is positioned at MACHINE Home location).*

(CIRCULAR AND RECTAGULAR POCKET “CANNED” CYCLES)

G 77 (CIRCULAR POCKET)

Format: G77 X Y H Z D A B I K S

X= CENTER POINT OF POCKET

Y= CENTER POINT OF POCKET

H=START HEIGHT   Z=MINUS TOTAL DEPTH

D=DIAMETER OF POCKET	A=STEP OVER

B=MINUS DEPTH OF CUT 	I=ROUGH FEED

K= FINISH FEED			S=FINISH STOCK

G78 (RECTANGULAR POCKET )

Format: G78 H Z X Y U A B I J K S

H=START HEIGHT	Z MINUS TOTAL DEPTH

X=LENGTH OF POCKET IN “X” DIRECTION

Y= LENGTH OF POCKET IN “Y” DIRECTION

U=CORNER RADIUS 	A=STEP OVER

B=MINUS DEPTH CUT	I=RAMP FEED

J=ROUGH FEED		K=FINISH FEED

S=FINISH STOCK

** Must have a position block to position cutter to center of rectangle prior to calling this canned cycle **

(DRILLING CANNED CYCLES)

G80	DRILLING CYCLE OFF

G81	BASIC DRILL (Feed,down to depth- rapid up)

Format: G81 Z R F P

Z=MINUS TOTAL DEPTH

R=STARTING HEIGHT .100”

F=FEED IN INCHES /MINUTE

P=RETURN HEIGHT

G83	PECK CYCLE (Feed down peck increment,  rapid up…repeat until final depth reached.

Z=MINUS TOTAL DEPTH

R=STARTING HEIGHT .100”

F=FEED IN INCHES \MINUTE

I=PECK DEPTH IN MINUS  -.05”

P=RETURN HEIGHT

G87 	CHIP BREAK CYCLE (Feed down peck increment, retract (U)…repeat until final depth reached

Z=MINUS TOTAL DEPTH

R=STARTING HEIGHT   .100

F= FEED IN INCHES /MINUTE

I=DEPTH OF FIRST PECK  -.050”

J=PECK DECREASES

K=MINI PECK(SMALLEST)

W=CHIP BREAK IN INCREMENTS

U=RETRACTING DEPTH

P=RETURN HEIGHT