COMMON FADAL CODES

Note:

- Be sure to use all CAPITAL letters

- START ALL PROGRAMS WITH % AND END WITH %

- (Letter-O) Program number(Must be placed on its own line right after first percent sign)

Example:	%

O6 (the 6 represents the program number)

(G CODES)

G0 RAPID MOTION

G1 LINEAR MOTION

G2 CLOCKWISE ARC

G3 COUNTERCLOCKWISE ARC

G28 RETURN TO ZERO OR HOME POSITION X0 Y0

G70 INCH mode

G71 METRIC mode

G90 ABSOLUTE (Cartesian point system)

G91 INCREMENTAL (Delta move from point to point

(CUTTER COMPENSATION)

(Should be invoked in a movement block, moving cutter as perpendicular as possible into the cut geometry.)

G40 CANCEL CUTTER COMPENSATION

G41 CUTTER-COMPENSATION= LEFT (CLIMB CUT)

G42 CUTTER-COMPENSATION= RIGHT (CONVENTIONAL CUT)

ADDITIONAL CODES

F= FEED IN INCHES PER MINUTE

H= TOOL LENGTH OFFSET MUST BE IN TOOL PAGE

D= DIAMETER OF TOOL MUST BE IN TOOL PAGE

S= SPEED OF SPINDLE

T= TOOL NUMBER

(M FUNCTIONS)

M2=END OF PROGRAM

M3=SPINDLE CLOCKWISE

M4=SPINDLE COUNTERCLOCKWISE

M5=SPINDLE OF

M6=TOOL CHANGE

M8=COOLANT ON

M9=COOLANT OFF

M17=END OF SUBROUTINE

M30=END OF SUBROUTINE SECTION

(DRILLING CYCLES)

G83 (PECK DRILL) (FOR DRILLING THRU)

G83 G98 R0+.1 Z-.6 F10.  I.1 J .05 K.05 P .1

or

G83 G98 R0+.1 Z-.6 Q.1  X?  Y? F10.

G81 (SPOT DRILL) (FOR CENTER DRILL)(REAMING)

G81 G98 R0+.1 Z-.6 F10

G80 CANCEL DRILL CYCLE

FIXED SUBROUTINES (CANNED CYCLES)

NOTE: LOCATE ALL FIXED  SUB ON EXACT LOCATION X? Y?

L93XX  BOLT CIRCLE  R0  R1  R2

R0=X DIRECTION & DISTANCE FROM CENTER

R1=Y DIRECTION &DISTANCE FROM CENTER

R2=ANGULAR STEP BETWEEN HOLES

PLUS += CCW		MINUS-= CW

XX= NUMBER OF HOLES

L94XX R0   R1  COUNTERCLOCKWISE

SET TOOL TO PROPER DEPTH Z-?

R0=FEED RATE

R1=DIAMETER OF HOLE TO BE BORED

XX= NUMBER OF TIME TO REPEAT

L95XX R0 R1 CLOCKWISE

SAME AS L94XX

L9601 R0  R1  R2  R3   CLIMB MILL

R0=FEED RATE

R1=STEP OVER  70% TOOL DIAMETER

R2=X WIDE OF POCKET

R3= Y LENGTH OF POCKET

L9701 R0  R1  R2  R3    CONVENTION MILL

SAME AS L 9601

L9801 R0+  R1+  R2+

# CIRCULAR POCKET CLEAN OUT CCW

R0=FEED RATE

R1=STEP OVER

R2=POCKET DIAMETER

L9901 R0+  R1+    R2+

SAME AS L9801 CLOCKWISE CLEAN OUT

(PROGRAM COMMENTS)

Use parenthesis around your comments

Example: (JOHN SMITH LAB-4)

*** Note: Do Not use # sign ***