# OUTLINE FOR FADAL

## 1. INTRODUCTION TO FADAL PROGRAMMING

- PUT % AT BEGINNING AND END

- ALL EVENTS MUST BE NUMBERED IN ORDER

- #1 EVENT HAVE O ? ( NAME OF PROGRAM

- G0 RAPID MOTION G1 FEED MOTION

- SUBROUTINES AT BEGINNING NOT END

- RAMP ON DIFFERENCE

2. DISCUSS NEW TERMS M, S, H, D. 			#24 MANUAL

3. DISCUSS RAMP ON MOVE AND DIFFERENCE WITH ANILAM

#23 DRAWING

#3 ANILAM RAMP ON

4.PROGRAMMING ELEMENTS

1.SUBROUTINES

### 1. Lnn00 NUMBER OF SUB (nn= 01-99)

2. Lnn## Sub repeat amount (##=01-99)

Ex: To name a sub: L100 is sub #1

Ex: To call the sub once:  L101

Ex: To call  the sub three times: L103

3. LAST EVENT G80 M5

4. END M17, END OF ALL SUB’S M30

5.AFTER M30 START CANNED CYCLES

#24 MANUAL

2.CANNED CYCLES

- M6 T#

- ( TOOL DESCRIPTION

- G0 G90 G70 S#### M3

- H#  Z.100 M7

- G81,G83 DRILL CANNED CYCLE

- L#0# LOCATION OF HOLES TO BE DRILLED

#24 MANUAL

3.FINAL PROGRAM FOR LETTERS

DISCUSS WHOLE PROGRAM

#26 PROGRAM OUTLINE