PAGE 1

ENGRAVING INFORMATION

N1 O1 *SAMPLE ENGRAVING PROGRAM

N2 M6 T3  (TOOL CHANGE,TOOL NUMBER)

N3 (TOOL #3 ENGRAVING TOOL .015 DIA

N4 G0 G90 S10000 M3 E1 X.375 Y.617

G0=RAPID,G90=ABSOLUTE, SPEED,M3=CW SPINDLE, E1=ENGRAVING ON

X=LOCATION OF FIRST LETTER

Y= LOCATION OF FIRST LETTER  MINUS ½ TOOL DIAMETER

N5 H3 D3   Z.1

H= TOOL LENGTH OF #3, D= #3 TOOL,

Z =.1  FIRST MOVE FOR TOOL #3

N6 L9201 R0, R1, R2, R3, Z-? F20.0

L9201= ENGRAVING  CANNED CYCLE

Z= ½ DIAM. OF BALL END MILL

F= FEED IN INCHES PER MINUTE

R0=CLEARANCE ABOVE PART FOR MOVING

R1=MODE OF LETTERS (SHAPE OF LETTERS)

R2=TOTAL HEIGHT OF CHARACTER

R3=POSSIBLE ANGLE OF LETTERS

R4= ONLY FOR SERALIZATION OF ENGRAVING

N7 M5  (SPINDLE OFF,

N8 G90 G0 H0 Z0

G90= ABSOLUTE, G0=RAPID, H0=CANCEL T.L.O. Z0= HOME

N9 E0 X0 Y0

E0= ENGRAVING OFF X0= HOME  Y0= HOME

N10 M2   M2= END OF PROGRAM

PAGE2

CALCULATING WORD LENGTH

STEP 1: ESTABLISH A DESIRED HEIGHT OF THE LETTERS

STEP 2: CHOOSE THE DIAMETER OF TOOL

FORMULAR=  .14 TIMES THE HEIGHT

STEP 3: CALCULATE THE LENGTH FACTOR USING

THE CHART ADDING ALL THE WIDTH FACTORS TOGETHER

STEP 4: THE ACTUAL LENGTH IS

(HEIGHT MINUS TOOL DIAM.)TMES (TOTAL LENGTH FACTOR)

LENGTH = (H-D)x( LF)

WIDTH OF CHARACTER BOX

WIDTH =(HEIGHT-DIAMETER)x(WIDTH FACTOR)

Wd = (H-D)x(WF)

START AND END FACTORS

SPACE = (HEIGHT-DIAMETER)x (SPACE FACTOR)

Sp =(H-D)x(SF)

Example:

Step 1:  Select .5”

Step 2: .125” endmill

Step 3:  Count letters (max 8).  Add the width factor for each letter from table and get total (‘Actual Length’).

Step 4:  Multiply Actual Length by (.5 – .125 = .375) This is the total width of the name.

Subtract this value from length of the PART.  Divide by two.   This is the X Factor in the program.

(Width of part - .5) and divide by two, then add .5  This is the Y Factor for the program (negative in our case).